| Sad Microcontrôleur ! |  |  |  |
| --- | --- | --- | --- |
| Capacités exigibles : | A | B | C |
| - Utiliser un dispositif comportant un microcontrôleur pour produire un signal sonore. - Relier qualitativement la fréquence à la hauteur d’un son audible. |  |  |  |

Document 1 :

La Carte à microcontrôleur Arduino®

| Document 2 : | Document 3 : |
| --- | --- |
| Une carte Arduino® est une carte munie d’un microcontrôleur. Il est possible de connecter à cette carte des capteurs pour réaliser des mesures (de température, de pression…) ou des actionneurs c’est-à-dire des composants qui agissent dans notre environnement : un moteur, un buzzer, une DEL.... Une carte à microcontrôleur peut être programmée. | buzzer |

Document 4 :

Document 5 :

http://jeanjacques.dialo.free.fr/frequenc.htm

|  | Défi : Jouer « Sad Trombone » à l’aide d’un microcontrôleur | https://www.youtube.com/watch?v=rRgNYN5H82M |
| --- | --- | --- |

|  | # tone() Generates a square wave of the specified frequency on a specified pin ### Syntax tone(pin, frequency, duration) |
| --- | --- |
| Only one tone can be generated at a time. If a tone is already playing on a different pin, the call to tone() will have no effect. If the tone is playing on the same pin, the call will set its frequency |  |

|  | # delay(ms) Pauses the program for the amount of time (in milliseconds) specified as parameter. ### Syntax delay(ms) |
| --- | --- |
| ms: the number of milliseconds to pause. |  |

|  | # pinMode() Configures the specified pin to behave either as an input or an output. Used in a setup function. ### Syntax pinMode(pin, mode) |
| --- | --- |
| pin: the Arduino pin number to set the mode of. mode: INPUT, OUTPUT |  |

|  | # setup() The setup() function is called when a sketch starts. Use it to initialize variables, pin modes, start using libraries, etc. ### Syntax void setup{ … } |
| --- | --- |
| The setup() function will only run once, after each powerup or reset of the Arduino board. |  |

|  | # loop() Use it to actively control the Arduino board. ### Syntax void loop{ … } |
| --- | --- |
| After creating a setup() function, which initializes and sets the initial values, the loop() function does precisely what its name suggests, and loops consecutively, allowing your program to change and respond. |  |

void setup() {

pinMode(pin, OUTPUT);

}

void loop() {

tone(pin,frequence);

delay(ms);

tone(pin,frequence);

delay(ms);

tone(pin,frequence);

delay(ms);

tone(pin,frequence);

delay(ms);

}

Programme

void setup() {

pinMode(8, OUTPUT);

}

void loop() {

tone(8,587);

delay(500);

tone(8,554);

delay(500);

tone(8,523);

delay(500);

tone(8,494,750);

delay(2000);

}

Partition « Sad Trombone »

D C# C B 		soit 			Ré Do# Do Si

Vidéo

How To Play Sad Trombone

https://www.youtube.com/watch?v=rRgNYN5H82M